FLOOD VULNERABILITY & SITE SUITABILITY ASSESSMENT: LEKKI-EPE CORRIDOR

Date: February 2026

Analyst: Eniola Akinnibi

1. EXECUTIVE SUMMARY

This report presents a geospatial risk assessment of the Lekki-Epe corridor, a rapidly urbanizing zone in Lagos State characterized by complex coastal hydrology. Using Multi-Criteria Decision Analysis (MCDA), this study models hydrostatic flood susceptibility to guide real estate investment and urban planning.

The analysis reveals that 35% (499 km2) of the study area is designated "Critical Vulnerability," primarily concentrated in the Ibeju-Lekki wetlands and lagoon buffers. Conversely, only 28% (398 km2) is classified as "High Suitability" (Green Zones), offering safe ground for development without intensive hydraulic engineering.

2. METHODOLOGY & DATA

The assessment adheres to a zero-cost, open-source intelligence (OSINT) framework, ensuring reproducibility and scalability.

- Data Sources:

  - Terrain: Copernicus GLO-30 DEM (30m resolution, hydro-conditioned).

  - Land Cover: Sentinel-2 Multispectral Imagery (10m resolution, ESA).

  - Infrastructure: OpenStreetMap (OSM) Road Networks.

- Analytical Model (MCDA):

A weighted overlay model was developed using the Analytical Hierarchy Process (AHP). Factors were weighted based on hydrological impact:

  - Topographic Wetness Index (35%): Modelling natural flow accumulation.

  - Distance to Water Bodies (30%): Euclidean distance buffering from Lagoon/Ocean.

  - Slope (20%): Identifying stagnation zones (0-2% gradient).

  - NDVI (15%): Assessing surface permeability.

3. ANALYSIS OF RESULTS

The spatial distribution of risk is non-uniform. The corridor is not a single landmass but a series of sandy ridges separated by drainage basins.

- Zone A: Critical Vulnerability (Risk Score 7-10)

  - Coverage: 499.41 km2 (34.8%)

  - Characteristics: These areas are natural drainage basins. Construction here requires significant sand-filling (>2m) and canalization.

  - Key Locations: Deep wetland strips behind the Refinery axis and the immediate Lagoon setbacks.

- Zone B: Moderate Suitability (Risk Score 4-7)

  - Coverage: 539.46 km2 (37.5%)

  - Characteristics: Transitional zones. While not marshes, these areas have high water tables or poor runoff coefficients. Development here requires mandatory plinth elevation (600mm+) and robust drainage networks.

- Zone C: High Suitability (Risk Score 1-4)

  - Coverage: 397.78 km2 (27.7%)

  - Characteristics: Elevated sandy ridges. Optimal for residential and commercial development with minimal risk mitigation costs.

4. STRATEGIC RECOMMENDATIONS

- Investment Focus: Capital deployment should prioritize the 28% "Green Zone" corridor, where asset security is naturally higher.

- Engineering Mandates: The 37% of land in "Moderate" zones must not be treated as dry land; it requires specific drainage bylaws to prevent future urban flooding.